An Efficient Active and Reactive Power Control of DFIG for wind power generator

1st  Sabah Louarem

Department of Electrical Engineering

Laboratory DAC HR

Ferhat Abbas University Setif 1, Algeria

slouarem@univ-setif.dz

3rd Tarek bouktir

Department of Electrical Engineering, Ferhat Abbas University Setif1, Algeria

tbouktir@univ-setif.dz

2nd Djamel Eddine chouib Belkhait

Department of Electrical Engineering

Laboratory DAC HR

Ferhat Abbas University Setif 1, Algeria

djamel.belkhait@univ-setif.dz

4th Saâd Belkhait

Department of Electrical Engineering

Laboratory DAC HR

Ferhat Abbas University Setif 1, Algeria

sbelkhait@univ-setif.dz

Abstract— During the past decade, wind energy become the most important energy sources in power systems but the disadvantage is that wind power generation is intermittent, depending on weather conditions.  The use of DFIG energy is gaining the most widely concept types of renewable energy generations because of several benefits: offer to reduce stresses of the mechanical structure, noise reduction and the possibility to control active and reactive power. This paper presents a comparative study on the performance of different control strategies for DFIG wind turbines: proportional–integral (PI), Artificial Neural Network 𝐴𝑁𝑁,H∞, and adaptive Fuzzy PI controller. As compared with obtained results, the simulation results show that the excellent performance, dynamic response, and robustness against machine parameters variations of DFIG is improved with H∞control technique.

Keywords-Wind energy, DFIG, PI controller, ANN Controller, H∞  Controller, Adaptive Fuzzy PI controller.

Introduction

All standard paper components have been specified for thre In the last few years, severe environmental problems such as global warming, air pollution caused by the extensive use of fossil fuels have attracted the world’s attention to renewable energy utilization [1]. Renewable energy sources like solar, biomass and wind are locally available sources of energy, which are safe and reliable for the environment [2]. Among them, the wind power, as a clean source of energy that can be converted to electricity by a wind energy conversion system (WECS), has received a considerable attention [3].

In fact, wind energy conversion system can be classified into two categories: variable and fixed-speed WECS. Among both techniques, variable speed wind generators are frequently used compared to fixed-speed systems because of their efficient energy production, improved power quality and dynamic performance during grid faults [4]. In this context, the doubly-fed induction generator (DFIG)has proved its worth as a powerful solution to improve the power rating of wind.

In the last few years, severe environmental problems such as global warming, air pollution caused by the extensive use of fossil fuels have attracted the world’s attention to renewable energy utilization [1]. Among them, the wind power, as a clean source of energy that can be converted to electricity by a wind energy conversion system (WECS), has received a considerable attention [3].

In fact, wind energy conversion system can be classified into two categories: variable and fixed-speed WECS. Among both techniques, variable speed wind generators are frequently used compared to fixed-speed systems because of their efficient energy production, improved power quality and dynamic performance during grid faults [4]. In this context, the doubly-fed induction generator (DFIG)has proved its worth as a powerful solution to improve the power rating of wind turbines. Hence, DFIG has become one of the most popular wind power generation systems owing to its high efficiency under variable wind speeds, four quadrant active and reactive power capabilities, and relatively low cost of power electronic converters [4-5].

Moreover, the control of DFIG-based wind energy conversion system has attracted a lot of attention. Thus, several control algorithms have been proposed to improve the DFIG-based wind turbine system behavior during normal operation [6]. All these control laws were generally based on vector control concept (with stator flux or voltage orientation). For instance, decoupled control of active and reactive power in a DFIG-based wind energy conversion system with conventional PI controllers has been proposed in [7]. However, the main disadvantage of this kind of controllers lies in the fact that its performance largely depends on the PI parameters tuning. This latter is conditional on accurate knowledge of machine's parameters such as stator and rotor resistances (by warming-up) and inductances (by saturation) which may degrade according to physical conditions [8]. To address this issue, several studies focused on the advanced and robust controllers have been performed. For example, Predictive Functional Control (PFC) was proposed in [9]. The tests have performed in increasing the rotor resistance by 100%, all in reducing the incidence of machine's parameters variations, in order to show the PFC's effectiveness compared to PI controller [9]. In the same context, a predictive power controller synthesized by using a linearized state-space model of DFIG has been proposed in [10]. In this study, the rotor resistance and the mutual inductance are increased by 20% in order to test the controller's robustness against parameter variations. Moreover, the design of an Adaptive Fuzzy Gain Scheduling of Proportional Integral controller (AFGPI) has been performed in [11] for to address the issue of changes in the parameters. Satisfactory power response was obtained, compared to the conventional PI controller, under variations of the rotor resistance Rr (+50%) , the stator 's and rotor's inductance and the mutual self-inductance (+20%). Otherwise, a neuronal controller has been designed in [12] to control the rotor currents of the DFIG. The robustness of the proposed controller has been tested when the stator and the rotor self-inductances increased by +20%. Another robust H∞  control design, based on a vector control strategy, for the DFIG system has been carried out in [13] for the purpose of controlling the stator active and reactive power flow between the DFIG and the grid. The tracking control performance of the proposed controller was evaluated by modifying the machine's parameters as follows: the rotor resistance was increased by 100% of its nominal value and that one of the mutual inductance is decreased by 10% of its nominal value. Obtained simulation results were satisfactory, therefore showing the robustness of the H∞ compared to the conventional PI controller. Recently a robust power command based on the Active Disturbance Rejection Control (ADRC) for DFIG used in WECS system has been designed in [14]. Comparative study between the proposed controller and a robust polynomial (RST) controller has been performed under the following variations of machine's parameters: the rotor resistance was increased by 50% of its nominal value and the values of the stator's and rotor's inductances were decreased by 10% of their nominal value. Simulation results have shown that the proposed ADRC controller has the best performances in terms of robustness against machine's parameters variations.

In this study, we investigate the control design problem for DFIG-based wind energy conversion. The DFIG's stator is directly connected to the grid and the rotor is connected to the grid via the machine's and grid's inverters. Four kinds of controllers are proposed for the purpose of driving the power flowing between the stator of DFIG and the grid. Comparatively with existing literature, the main contribution of this work is to carry out a deep comparative study on the effectiveness of the more commonly used methods for DFIG's control, namely PI controller, Artificial Neural Network ANN controller, robust H∞ controller and adaptive fuzzy PI controller.

The remainder of this paper is organized as follows. Section 2 states the wind turbine conversion system modeling, followed by the modeling of the DFIG. The synthesis approaches of the proposed controllers are presented in section 3. Deep simulations and comparative study are illustrated in section 4. The paper ends with conclusions and cited references.

Modeling of wind turbine conversion system

Corresponding author: Sabah Louarem

The diagram of the DFIG-based wind turbine is shown in Figure 1, where (RSC) and (GSC) are, respectively, the rotor side converter and the grid side converter.

Aeroturbine

rotor

Wind

Filter

3ph, 50

Ωtur

Ωmec

Ps

Pr

ir d,q_mes

AC

DC

DC

AC

Control system

Mesured

Variables

References

Values

DFIG

Grid side

Converter

GSC

Rotor side

Converter

RSC

DC Bus

Basic configuration of a DFIG wind turbine.

A. Wind turbine model

The fraction of power extracted from the wind by a wind turbine is usually referred by the symbol Cp, which stands for the coefficient of performance or power coefficient. The actual mechanical power output Pm of a wind turbine is given as follows [15-16]:

Pm=12.ρ.π.R2Cpλ,βvwind3                   (1)

where, Pm  is the mechanical output power of the turbine [W]; ρ is the air density [Kgm3]. R is the rotor radius [m]; β  is the blade pitch angle [deg].

The power coefficient expresses the rotor aerodynamics as a function of both tip speed ratio λ and the pitch angle of the rotor blades  β. The tip speed ratio is defined as the ratio between the blade tip speed and the wind speed, expressed as [17]:

λ=ωRvwind                                 (2)

where ω is the rotor speed [rad] and vwind is the wind speed [m/s].

In order to determine the power coefficient Cp, the following well-know generic equation is used [18-19]:

Cp=0.5-0.0167.β-2.sinπ.λ+0.118.5-0.3β-2-0.00184.λ-3.β-2        (3)

turbine Cp curves for several values of β.

- B. Modeling of the doubly-fed induction generator

The application of Concordia and Park’s transformation to the three-phase model of the DFIG allows us to write the dynamic voltages and fluxes equations in an arbitrary d–q  reference frame [20-21]. The stator voltage equations are

Vsd=RsIsd+ddtφsd-ωsφsqVsq=RsIsq+ddtφsq-ωsφsd                  (4)

The rotor voltage equations are

Vrd=RrIrd+ddtφrd-ωs-ωrφrqVrq=RrIrd+ddtφrq-ωs-ωrφrd   (5)

The stator flux linkage equations are

φds=LsIds+LmIdrφqs=LsIqs+LmIqr                                    (6)

The rotor flux linkage equations are

φdr=LrIdr+LmIdsφrq=LsIrq+LmIsq                                      (7)

The active and reactive powers at the stator side and rotor side of DFIGcan be given as follows:

Ps=VsdIsd+VsqIsqQs=VsqIsd-VsdIsq                                    (8)

Pr=VrdIrd+VrqIrqQr=VrqIrd-VrdIrq                                   (9)

The electromagnetic torque is expressed as a function of the number of pole- pairs p:

Tem=-32pLmLsφsdIrd-φsqIsd                (10)

The stator current of DFIGwas decomposed into active and reactive component via the coordinate transformation and the field-oriented vector transformation.

In order to regulate the power of the DFIG based wind turbine system, the well-know vector control strategy is used. This one consists of using the flux orientation, such as stator flux orientation (SFO)is given as follows [7-22]:

Vsd=0Vsq=Vs=ωsφs                                      (11)

Active and reactive power of stator can then be expressed only versus these rotor currents as,

Sb

Rb

φds

Ra

d-q frame

Rotor Axis

Sa

Stator Axis

Rc

Sc

θr

θe

Vqs

θs

d

q

O

Stator and rotor flux vectors in the synchronous d-q Frame.

Ps=-VsLmLsIrqQs=VSψsLs-VsLmLsIrd                         (12)

Vrd=Rr.Ird+Lr-Lm2LsdIrddt-gωsLr-Lm2LsIrq    Vrq=Rr.Irq+Lr-Lm2LsdIrqdt+gωsLr-Lm2LsIrd+gωSLm.φsLs                                                     (13)

## In steady state, the second derivative terms in (13) are neglected and the third term constitutes cross-coupling tams.

Vrd=Rr.Ird-gωsLr-Lm2LsIrq    Vrq=Rr.Irq+gωsLr-Lm2LsIrd+gωSLm.φsLs     (14)

Controllers synthensis

A.  PI Controller synthesis

Elaboration of this controller is simple. Figure 4 shows the block diagram of the system controlled with PI  controller. The terms Kp and Ki represent respectively the proportional and integral gains. The ratio  BA represents the transfer function, to be controlled, where A and B are presently defined as follows:

A=LsRr+pLsLr-Lm2Ls and B=-LmVs     	(15)

The controller gains were obtained in this study by using the pole-compensation method. The response time of the controlled system will be fixed at 10 ms. This value is sufficient for our application and a lower value might involve transients with important overshoots. The calculated terms are given in [7-23].

Yref

-

kP+kip

BA

Y

+

-

System with PI controller.

The transfer function of the open loop including the controller is formulated as follows:

Gp=p+kikppkp.Lm.VsLrLs-Lm2p+LsRrLrLs-Lm2	             (16)

To cancel the pole, we added a zero at the same location as the pole, equation (16) gives a pole value.

kikp=LsRrLrLs-Lm2	      	                           (17)

The transfer function of the loop becomes:

Gp=-kpLmVsLrLs-Lm2p	                            (18)

The transfer function of the closed loop is expressed by:

Hp=11+pτr	                  	            (19)

τr: is the response time.

Now, we express the controller gains correction based on the settings of the machines and the response time as follows:

kp=-1τrLsLr-Lm2LsLmVski=-1τrRrLsLmVs 	                          (20)

B. ANN Controller synthesis

As it is well know, ANN models was largely inspired from the basic framework of the brain.  QUOTE ANNANN consists of many nodes and connecting synapses. The nodes operate in parallel and communicate with each other via connecting synapses [24]. Several complex tasks in various fields of application including pattern recognition, identification, classification, speech, vision and control systems, have been performed by using ANNs. However, the most widely used neural network algorithm in the control systems engineering is back-propagation. This algorithm attempts to minimize the error for each output neuron by adjusting the weights of the neurons using a gradient descent optimization algorithm. In the back-propagation learning, the actual outputs are compared with the target values to derive the error signals, which are propagated backward layer by layer for the updating of the synaptic weights in all the lower layers [25].

In this study, the ANN controller was performed by using the nftool of Matlab. The DFIG-based wind turbine is simulated with the PI controller. The data before and after the PI controller is computed and saved in the workspace. After that, the network training was done until the system undergoes 10 iterations. We note that the hidden layers contained 40 neurons.

Input

f

f

f

f

+

Hidden layers

Output

W11

Vrq

W12

b1

b2

b5

W15

W26

f

W57

f

-

W13

b7

W14

W36

W45

W67

b3

b6

f

W46

b4

W16

Pref

Pmes

Neural network structure.

C. Design of H∞ controller

In this section, we present H∞ mixed-sensitivity synthesis method for robust tracking control design. The main idea consists of synthesize a controller by minimizing an H∞ performance objective.

As mentioned above, the quotient BA represents the transfer function to be controlled. Thus, we can define the tracking error, the input control and the output as follow:

es=SsYrefs			  (21)

us=KsSsYrefs		            (22)

Ys=TsYrefs                                  (23)

where Ss=I+BsAsKs-1is sensitivity function and  Ts=BsAsKsI+BsAsKs-1is the closed-loop transfer function.

In the sequel, we are interested in designing of the controller Ks. For that, we will consider the H∞ mixed-sensitivity design problem illustrated in Figure 6.

BsAs

Ks

W1s

W2s

SKS Mixed-sensitivity minimization.

where, W1s and  W2s are selected as loop-shaping weights. Hence, the tracking control design is considered as an optimization problem which consists of finding a stabilizing controller Ks to minimize the following cost function:

W1sSsW2sKsSs∞                      (24)

The design weights W1s and W2s were selected as:

W1s=6000s+30	                              (25)

W2s=0.0025s20.0005s2+5s+100	      	 (26)

By using robust control toolbox of Matlab, we obtain the following controller:

(27)

D. Adaptive Fuzzy PI controller synthesis

The aim of this section is to present the design of an adaptive fuzzy PI controller which is able to ensure robust tracking performances of the DFIG  facing the parametric changes. The proposed scheme is given in figure 7. To improve the performance of the PI controller, its gains are designed using Fuzzy Logic System (FLS). The FLS  inputs are the errors (e) in active, reactive(e), and rate of change in active and reactive power error (de). The output of the fuzzy controller is (du).

In contrast to PI controller where controller parameters (kp and kigains) are tuned in a predefined way [30], FLS is used to tune the parameters kp and ki of PI so that the controller can keep up with parametric changes and reference signals can be tracked.

Yref

ddt

FLC

+

Y

Process

-

PI Controller

PI control system with fuzzy gain adapter.

The fuzzy sets are defined as follows: NB is Negative Big, NM is Negative Medium, NS is Negative Small, ZR is Zero, PS is Positive Small, PM is Positive Medium, PB is Positive Big, B is Big and S is small. The max-min reasoning method and the center of gravity defuzzification method are used [31].

Figures 8(a) represents the degree of membership functions for the error e and change of error de . Figure 8(b) represents the degree of the membership functions for the proportional gain Kp' and the integral gain  Ki' of the active and reactive power controller.

(a)                                                  (b)

(a) The error and its variation membership functions.

( b) The Kp'and Ki'membership functions.

The values of Kp' and Ki' are determined by a set of fuzzy rules of the form designed fuzzy gain scheduling controller has two inputs and two outputs.

Kp'=Kp-KpminKpmax-Kpmin	 		   (28)

Ki'=Ki-KiminKimax-Kimin	  		   (29)

This section is dedicated, on one hand, to illustrate the effectiveness of the proposed controllers to minimize tracking error and, on the other hand, to compare their performances DFIG against machine's parameter variations. For that, deep simulations are performed by using MATLAB environment. The proposed control algorithms of DFIG, used in this paper is 1.5 MW1.5 MW, whose nominal parameters are indicated in Appendix. Thus, it is first tested as in ideal conditions mode and driven to 1500 rpm. Then the different controllers are tested and compared by two different criteria’s; namely reference tracking performance, and robustness by varying the parameters of the system.

A. Reference tracking in ideal conditions mode

Simulation results of active and reactive power using classical PI controller, ANN controller, the proposed H∞H-infinityH-infinity controller and the proposed adaptive fuzzy PI controller are shown in figure 9. As expected, both active and reactive powers have been produced with good accuracy. Moreover, they can track the desired reference after a finite time interval and the answers are without overshoots for different controllers.

Active and reactive powers' behavior.

Zoom in active and reactive powers' behavior.

Moreover, to compare the response times of all controllers, the figure 10 shows a zoom view of active and reactive powers' behavior from 0 to 0.005s. Hence, we can conclude a quicker response time for ANN and H-infinityH∞ controllers. In the same context, we notice that the system controlled by PI controller has evolved to steady state with an important response time in comparison with the other controllers.

Evolution of cumulative tracking error functions.

In order to show which controller has best steady state accuracy, we graph the evolution of the following cumulative tracking error functions:

CTEP=0tPs-Psrefdt,            t∈02s                (30)

CTEQ=0tQs-Qsrefdt,            t∈02s             (31)

As is shown in figure 11, the H∞ controller has best steady state accuracy in comparison with the other controllers. In contrast to the H∞ controller, the conventional PI controller has low steady state accuracy.

B. Robustnes

In this subsection, we are interested by evaluating the robustness of the proposed controllers against the machine's parameters variation, such as the inductances Ls, Lr, the mutual inductance Lm and the resistance Rr. For that, deep simulations were performed using Matlab environment. Thus, the results of this study are compared according to the rise time, the maximum peak overshoot and settling time of each controller.

Now, we test the robustness of the proposed controllers against mutual inductance variations. Hence, the figure 12 and the table I present the simulation results for a parametric variation in the order of-10%. Unlike the Adaptive fuzzy PI controller and the H∞ controller, we can remark that the ANN controllers and PI controller lose its performances in terms of the maximum overshoot.

Active and reactive powers' behavior with -10% of mutual inductance variation.

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0102 | 29.43 | 0.0541 |
| ANN | 0.0018 | 7.62 | 0.0023 |
| H∞ | 0.0023 | 0.77 | 0.001 |
| Adaptive Fuzzy PI | 0.0024 | 0.01 | 0.0042 |

TABLE I.b

- Comparison of the response parameters of

- reactive powerLm-10%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0102 | 19.20 | 0.0252 |
| ANN | 0.0005 | 7.62 | 0.0021 |
| H∞ | 0.0005 | 1.20 | 0.0010 |
| Adaptive Fuzzy PI | 0.0024 | 0.14 | 0.0042 |

In order to refine our study, we test the robustness of the proposed controllers against a mutual inductance variation in the order of -25%. Hence, the simulation results are presented in the figure 14 and the table II. We can note that the PI and ANN controllers lose its performance in response time with undesirable maximum overshoots and only adaptive fuzzy PI controller and H∞ controller can be resisting this parametric variation with a slight superiority for H∞ controller in terms of response time.

Active and reactive powers' behavior using H∞H-infinity and  QUOTE PI-Adaptive Fuzzyadaptive fuzzy PI controllers with -25% of mutual inductance variation.

TABLE II.a

- Comparison of the response parameters of

- active  power Lm-25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0014 | 1.95 | 0.0057 |
| Adaptive Fuzzy PI | 0.0060 | 0.61 | 0.0104 |

TABLE II.b

- Comparison of the response parameters of

- reactive power Lm-25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0015 | 2.96 | 0.0058 |
| Adaptive Fuzzy PI | 0.0060 | 0.70 | 0.0106 |

In the same way, we test in the sequel the robustness of the proposed controllers against rotor inductance variations. Two deep simulations have been performed (figures 15 and 16). The first one concerns a variation in the order of +10% against a variation in the order of +25% for the second simulation. Thus, the results are reported in the tables III and IV. In this case, as in the preceding one, only two controllers have resisted the second test with slight superiority regarding the response time for the  H∞ controller.

Active and reactive powers' behavior with +10% of rotor inductance variation.

TABLE III.a

- Comparison of the response parameters of

- active power  Lr+10%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0066 | 11.94 | 0.0414 |
| ANN | 0.0011 | 0.37 | 0.0012 |
| H∞ | 0.0003 | 0.40 | 0.0005 |
| Adaptive Fuzzy PI | 0.0013 | / | 0.0023 |

TABLE III.b

- Comparison of the response parameters of

- reactive  power  Lr+10%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0060 | 17.76 | 0.0427 |
| ANN | 0.0003 | 17.16 | 0.0013 |
| H∞ | 0.0003 | 0.78 | 0.0005 |
| Adaptive Fuzzy PI | 0.0013 | 0.25 | 0.0024 |

Active and reactive powers' behavior using H∞H-infinity and PI-Adaptive Fuzzy adaptive fuzzy PI controllers with +25% -25% of rotor inductance variation.

TABLE IV.a

- Comparison of the response parameters of

- active power  Lr+25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0007 | 0.93 | 0.0011 |
| Adaptive Fuzzy PI | 0.0028 | 0.31 | 0.0106 |

TABLE IV.b

- Comparison of the response parameters of

- reactive power Lr+25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0007 | 1.54 | 0.0038 |
| Adaptive Fuzzy PI | 0.0028 | 0.18 | 0.0011 |

Regarding stator inductance variations, the simulation results have been reported in the tables V and VI. As in the preceding cases, only the H∞ andadaptive fuzzy PI controllers can be resisting the high stator inductance variation.

Active and reactive power behavior using the four controllers with +10% variation of Ls.

TABLE V.a

- Comparison of the response parameters of

- active powerLs+10%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0066 | 11.51 | 0.0411 |
| ANN | 0.0010 | 0.36 | 0.0012 |
| H∞ | 0.0003 | 0.39 | 0.0005 |
| Adaptive Fuzzy PI | 0.0024 | 0.01 | 0.0042 |

TABLE V.b

- Comparison of the response parameters of reactive

- power Ls+10%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.0068 | 17.11 | 0.0424 |
| ANN | 0.0003 | 18.86 | 0.0012 |
| H∞ | 0.0003 | 0.74 | 0.0005 |
| Adaptive Fuzzy PI | 0.0013 | / | 0.0023 |

Active and reactive power behavior using the H∞and Adaptive Fuzzy PI controller with +25% variation of Ls.

TABLE VI.a

- Comparison of the response parameters of

- active power  Ls+25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0007 | 0.90 | 0.0011 |
| Adaptive Fuzzy PI | 0.0028 | 0.01 | 0.0049 |

TABLE VI.b

- Comparison of the response parameters of

- reactive power Ls+25%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| H∞ | 0.0007 | 1.30 | 0.0011 |
| Adaptive Fuzzy PI | 0.0003 | 1.80 | 0.0052 |

Finally, regarding the rotor resistance variation, which is the result of the rotor heating, a deep simulation is performed by considering a rotor resistance variation in the order of 100%. The simulation results are shown in the figure 18 and the controllers' performances are summarized in the table VII. Thus, we can see that the different controllers can drive the system to track the reference signals. In other word, the active and reactive powers follow correctly the reference signals.

As far as the active power is concerned, it is clear that the H∞ H- infinity controller has the best rise time and settling time compared with other controllers. However, concerning the reactive power, the controllers' performances are almost identical to those of the ANN controller with a slight superiority for H∞  controller.

In a nutshell, the simulation results presented in this work have shown that the H∞ controller has the best steady state accuracy compared with other controllers. As regards the robustness against parametric variation, the H∞ and adaptive fuzzy PI controllers have the best tracking performances and robustness with a slight superiority for the H∞ controller.

Active and reactive power behavior using the four controllers

- with +100% variation of Rr.

TABLE VII.a

- Comparison of the response parameters of

- active power Rr+100%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.00220 | 1.17 | 0.02330 |
| ANN | 0.00020 | 0.10 | 0.00020 |
| H∞ | 0.00003 | 0.39 | 0.00007 |
| Adaptive Fuzzy PI | 0.0002 | / | 0.00050 |

TABLE VII.b

- Comparison of the response parameters of

- reactive power Rr+100%

| Controller | Rise time (s) | Maximum overshoot (%) | Settling time (s) |
| --- | --- | --- | --- |
| PI | 0.00240 | 5.15 | 0.1500 |
| ANN | 0.00005 | 24.37 | 0.1498 |
| H∞ | 0.00001 | 71.37 | 0.1499 |
| Adaptive Fuzzy PI | 0.00004 | 3.01 | 0.1500 |

In this paper, the control of doubly fed induction generator (DIFG) based wind energy conversion has been studied. The DFIG's stator was directly connected to the grid and the rotor was connected to the grid via the machine's and grid's inverters.

The design of four control strategies, namely PI controller, ANN controller, adaptive fuzzy PIcontroller and H∞ controller, has been presented and discussed.

The obtained results have revealed that the H∞controller is the best one to drive the DFIG's stator to track perfectly the active and reactive power references with very good tracking performance, namely low-rise time, low settling time and high steady state accuracy. Likewise, the H∞ controller seems more robust that all other controllers against machine's parameters variations.

References

- Yang, B., Zhang, X., Yu, T., Shu, H., Fang, Z,“Grouped grey wolf optimizer for maximum power point tracking ofdoubly-fed induction generator based wind turbine”,Energy conversion management. 133, 1, 427-443 (2017).

- Varzaneh, S.V., Abedi, M., Gharehpetian, G.B,“ A new simplified model for assessment of power variation of DFIG-based wind farm participating in frequency control system”, Electric Power Systems Research. 148, 220–229 (2017).

- Dileep Kumar Varma Sagiraju et all, Dynamic performance improvement of standalone battery integrated PMSG wind energy system using proportional resonant controller, Engineering Science and Technology, an International Journal, Volume 20, Issue 4, August 2017, Pages 1353-1365.

- Om Prakash Bharti, R. K. Saket , S. K. Nagar ,“Controller Design of DFIG Based Wind Turbine by Using Evolutionary Soft Computational Techniques”, Engineering, Technology & Applied Science Research Vol. 7, No. 3, 2017, 1732-1736.

- Heng Nian, Chenwen Cheng, and Yipeng Song, “ Coordinated Control of DFIG System based on Repetitive Control Strategy under Generalized Harmonic Grid Voltages”, Journal of Power Electronics, Vol. 17, No. 3, pp. 733-743, May 2017.

- V.N. Ananth Duggirala a, V. Nagesh Kumar Gundavarapu, Improved LVRT for grid connected DFIG using enhanced field oriented control technique with super capacitor as external energy storage system, Engineering Science and Technology, an International Journal 19 (2016) 1742–1752.

- Shehata, E.G,“Sliding mode direct power control of RSC for DFIGs driven by variable speed wind turbines”,Alexandria Engineering Journal. 54, 4, 1067-1075(2015).

- Poitiers, F., Bouaouiche, T., Machmoum, M,“ Advanced control of a doubly-fed induction generator for wind energy conversion”,Electric Power Systems Research. 79, 1085–1096 (2009).

- Taveiros, F.E.V., Barros, L.S., Costa, F.B,“Back-to-back converter state-feedback control of DFIG (doubly-fed induction generator)-based wind turbines”,Energy. 89, 896-906 (2015).

- Xin-fang, Z., Da-ping, X., Yi-bing, L,“Predictive Functional Control of a Doubly FedInduction Generator for Variable Speed Wind Turbines”,Proceedings of the 5th World Congress on Intelligent Control and Automation, June 15-19, 2004, Hangzhou. P.R. China.

- Sguarezi Filho, A. J., de Oliveira Filho, M. E., Ruppert Filho, E,“ A Predictive Power Control for Wind Energy”. IEEE Tansactions On stainable Energy. 2, 1, 97-105 (2011).

- Bedoud, K., Ali-rachedi, M., Bahi, T., Lakel, R,“Adaptive Fuzzy Gain Scheduling of PI Controller for control of the Wind Energy Conversion Systems”, Energy Procedia. 74, 211 – 225 (2015).

- Bedoud, K., Ali-rachedi, M., Bahi, T., Lakel, R., Grid, A,“ Robust Control of Doubly Fed Induction Generator for Wind Turbine Under Sub-Synchronous Operation Mode”,Energy Procedia. 74, 886 – 899 (2015).

- Abdelmalek, S., Belmili, H,“A New Robust H∞Control Power. Handbook of Research on Advanced Intelligent Control Engineering and Automation”,DOI: 10.4018/978-1-4666-7248-2.ch022 (2015).

- Boualouch, A., Nasser, T., Boukhriss, A., Frigui, A,“Robust Power Control of a DFIG used in Wind Turbine Conversion System”, International Energy Journal. 17, 1, 1-10 (2017).

- Ebrahimkhani, S,“ Robust fractional order sliding mode control of doubly-fed induction generator (DFIG)-based wind turbines”, ISA Transactions.63, 343-3540 (2016).

- Tapia, A., Tapia, G., Xabier Ostolaza, J., Ramón Sáenz, J, “Modeling and Control of a Wind Turbine Driven Doubly Fed Induction Generator”,IEEE Transactions On Energy Conversion. 18, 2, 194-204 (2003).

- Fernandez, L.M., Garcia, C.A., Jurado, F,“Comparative study on the performance of control systems for doubly fed induction generator (DFIG) wind turbines operating with power regulation”, Energy. 33, 1438– 1452 (2008).

- Abdin, E.S., Xu, W,“Control Design and Dynamic Performance Analysis of a Wind Turbine-Induction Generator Unit”. IEEE Transactions On Energy. 15, 1, (2000).

- El aimani, S,“Modelisation de differents technologies d'éoliennes intégrées dans un réseau de moyenne tension” ,Thèse de doctorat del'école centrale de LILLE et de l'université des sciences et technologie de LILLe. (2004).

- Ghennam , T., Aliouane, K., Akel, F., Francois, B., Berkouk, E.M,“Advanced control system of DFIG based wind generators for reactive power production and integration in a wind farm dispatching”, Energy Conversion and Management. 105, 240–250 (2015).

- Varzaneh, S.G., Abedi, M., Gharehpetian, G.B,“A new simplified model for assessment of power variation of
DFIG-based wind farm participating in frequency control system”, Electric Power Systems Research. 148, 220–229 (2017).

- Li,S., Wang , H.; Tian, Y.,  , Aitouch, A., Klein, J,“Direct power control of DFIG wind turbine systems based
on an intelligent proportional-integral sliding mode control”, ISA Transactions. 64, 431-439 (2016).

- Louarem, S., Belkhiat, S., Belkhiat, D.E.C,“A Control Method using PI/Fuzzy Controllers Based-DFIG in Wind Energy Conversion System”, IEEE Grenoble PowerTech, Grenoble (2013).

- Ata, R, “Artificial neural networks applications in wind energy systems”: a review. Renewable and Sustainable Energy Reviews, 49; 534–562 (2015).

- Alberdi, M.; Amundarain, M., Garrido, A., Garrido, I,“Neural control for voltage dips ride-through of oscillating water column-based wave energy converter equipped with doubly-fed induction generator”, Renewable Energy. 48, 16-26 (2012).

- Skogestad, S., Postlethwaite, I,“Multivariable feedback control: analysis and design”, JOHN WILEY & SONS, New York, NY, USA (2007).

- Khoete, S., Manabe, Y., Kurimoto, M., Funabashi, T., Kato, T,“Robust H-infinity Control for DFIG to Enhance Transient Stability during Grid Faults”, Proceeding of the World Congress on Engineering and Computer Science (WCECS), San Francisco, USA (2016).

- Duc, G., Font, S,“Commande H∞ et μanalyse des outils pour la robustesse”, HERMES Science publications, PARIS (1999).

- Brahim, M., Bahri, I., Bernard, Y,“Real time implementation of H infinity and RST motion control of rotary traveling wave ultrasonic motor”, Mechatronics. 44, 14–23 (2017).

- Zhao, Z. Y., Tomizuka, M., Isaka, S,“Fuzzy Gain Scheduling of PID Controllers”, IEEE Trans. on Systems, Man. and Cybernetics. 23, 5, 1392-1398 (1993).

- Ghani, A., Tahour, A.A., Abid. M., Essounbouli, N., Nollet, and F, “A Fuzzy-PI control technique designed for Power Control of Wind Turbine based on Induction Generator”, Advanced Materials Research. 875-877, 1676-1682 (2014).